STEINEL Professional Solar light

XSolar GL-S

Anthracite

Dimensions (L x W x H): 188 x 194 x 1178 mm;With lamp: Yes, STEINEL LED system;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Potentiometers;Version: Anthracite;PU1, EAN: 4007841085681;Application, place: Outdoors;Application, room: outdoors, garden, terrace / balcony, courtyard & driveway;Colour: Anthracite;Installation site: Stand mounted;Impact resistance: IK03;IP-rating: IP44;Protection class: III;Ambient temperature: from -20 up to 40 °C;Housing material: Aluminium;Cover material: Plastic, transparent;Power supply, detail: Solar supply, no mains power required, Lithium-iron-phosphate rechargeable battery 2500 mAh;Sneak-by guard: Yes;Capability of masking out individual segments: No;Reach, radial: r = 1.5 m (3 m²);Reach, tangential: r = 5 m (31 m²);Photo-cell controller: Yes;Luminous flux total product: 150 lm;Measured luminos flux (360°): 150 lm;Colour temperature: 3000 K;Colour Rendering Index: = 82;Lamp: LED cannot be replaced;LED life expectancy (max. °C): 50000 h;Service life LED L70B50 (25°): > 60000 h;Drop in luminous flux in accordance with LM80: L70B10;LED cooling system: Passive Thermo Control;Soft light start: No;Functions: Motions sensor;Twilight setting: 2 lx;Time setting: 10 sec – 0,5 min;Basic light level function: Yes;Basic light level function time: all night;Basic light level function in per cent: 3 %;Basic light level function percentage, from: 3 %;Basic light level function percentage, up to: 3 %;Output: 1,2 W;Battery charge: ab - 20°;Akku-Funktion: ab - 20;Detection angle: 140 °;Product category: Solar light

Manufacturer Steinel

Prod. No. 085681

Ordering designation XSolar GL-S Anthracite

Deliver, install and set ready for operation