STEINEL Professional Sensor-switched LED indoor light

RS PRO A10 SC

warm white white

Dimensions (Ø x H): 193 x 69 mm;With lamp: Yes, STEINEL LED system;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Bluetooth, App;With remote control: No;Version: warm white white;PU1, EAN: 4007841088316;Application, place: Indoors;Application, room: corridor / aisle, function room / ancillary room, warehouse, kitchenette, stairwell, changing room, WC / washroom;Colour: white;includes sheet of self-adhesive numbers: No;Installation site: ceiling, wall;Installation: Surface wiring, Ceiling;Impact resistance: IK10;IP-rating: IP54;Protection class: II;Ambient temperature: from -20 up to 40 °C;Housing material: Aluminium;Cover material: PC, opal;Mains power supply: 220 – 240 V / 50 – 60 Hz;Mounting height max.: 4,00 m;HF-system: 5,8 GHz;Slave modeselectable: Yes;Detection: also through glass, wood and stud walls;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: Ø 8 m (50 m²);Reach, tangential: Ø 8 m (50 m²);Photo-cell controller: Yes;Transmitter power: < 1 mW;Luminous flux total product: 1095 lm;Total product efficiency: 102,3 lm/W;Colour temperature: 3000 K;Colour variation LED: SDCM3;Lamp: LED cannot be replaced;Base: without;LED cooling system: Passive Thermo Control;Soft light start: Yes;Continuous light: selectable;Functions: Selecting programmed lighting scenarios, Motions sensor, DIM function, Adjustable fade time when switching on and off, Semi-/fully automatic, Neighbouring group function, Encrypted communication, Timer;Twilight setting: 2 – 2000 lx;Time setting: 10 sec – 60 min;Basic light level function: Yes;Basic light level function time: 1-60 min;Main light adjustable: 0 - 100 %;Twilight setting TEACH: Yes;Interconnection: Yes;Type of interconnection: Master/master;Interconnection via: Bluetooth Mesh;Lifetime LED according to IEC-62717 (L70): > 60.000 Std.;Basic light level function in per cent: 0 – 100 %;Output: 10,7 W;Colour Rendering Index CRI: = 80;Photobiological safety in accordance with EN 62471: RG1;Angle of aperture: 160 °;IP-rating, ceiling: IP54;Detection angle: 360 °;Product category: Sensor-switched LED indoor light

Manufacturer Steinel

Prod. No. 088316

Ordering designation RS PRO A10 SC warm white white

Deliver, install and set ready for operation