STEINEL Professional Motion detector

MD IR N360

COM1 - in-ceiling installation anthracite

Dimensions (Ø x H): 83 x 74 mm;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Bluetooth, Smartphone, Tablet;With remote control: No;Version: COM1 - in-ceiling installation anthracite;PU1, EAN: 4007841087432;Type: Motion detectors;Application, place: Outdoors, Indoors;Application, room: front door, courtyard & driveway, all round the building, terrace / balcony, Indoors, outdoors;Colour: Anthracite;Colour, RAL: 7016;Includes corner wall mount: No;Installation site: ceiling;Installation: Ceiling, In-ceiling installation;IP-rating: IP54;Protection class: II;Ambient temperature: from -20 up to 40 °C;Material: Plastic;Mains power supply: 220 – 240 V / 50 – 60 Hz;Switching output 1, resistive: 2000 W;Switching output 1, number of LEDs / fluorescent lamps: 10 pcs.;Power consumption: 0,3 W;Technology, sensors: passive infrared, Light sensor;Mounting height: 1,80 – 6,00 m;Mounting height max.: 6,00 m;Optimum mounting height: 2 m;Detection angle: 360 °;Sneak-by guard: Yes;Capability of masking out individual segments: Yes;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: Ø 7 m (38 m²);Reach, tangential: Ø 26 m (531 m²);Switching zones: 1179 switching zones;Cover material: shrouds;Functions: Motions sensor, Light sensor, Relay output, Data monitoring;Twilight setting: 2 – 2000 lx;Time setting: 5 sec – 60 min;Basic light level function: No;Twilight setting TEACH: Yes;Constant-lighting control: No;Interconnection: Yes;Interconnection via: Bluetooth Mesh;Reichweite Radial Eagle Eye max.: 6 m;Product category: Motion detector

Manufacturer Steinel

Prod. No. 087432

Ordering designation MD IR N360 COM1 - in-ceiling installation anthracite

Deliver, install and set ready for operation