STEINEL Professional Presence detector

Hallway

COM2 - Surface wiring

Dimensions (L x W x H): 62 x 123 x 123 mm;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Bluetooth Mesh;With remote control: No;Version: COM2 - Surface wiring;PU1, EAN: 4007841086343;Type: Presence detector;Application, place: Indoors;Application, room: Indoors, corridor / aisle;Colour: white;Colour, RAL: 9003;Includes corner wall mount: No;Installation site: ceiling;Installation: Surface wiring, Ceiling;IP-rating: IP54;Ambient temperature: from -20 up to 50 °C;Material: Plastic;Mains power supply: 220 – 240 V / 50 – 60 Hz;Switching output 1, resistive: 2000 W;Switching output 1, number of LEDs / fluorescent lamps: 8 pcs.;Switching output 1, floating: No;Switching output 2, resistive: 230 W;Switching output 2, floating: Yes;Switching output 1, low-voltage halogen lamps: 2000 VA;Power consumption: 0,7 W;Technology, sensors: High frequency, Mixed light metering;Voltage range (min): 220 V;Voltage range (max): 240 V;Mounting height: 2 – 4 m;Mounting height max.: 4,00 m;Optimum mounting height: 2,8 m;HF-system: 5,8 GHz;Detection: also through glass, wood and stud walls;Detection angle: Aisle, 360 °;Angle of aperture: 160 °;Sneak-by guard: Yes;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: 25 x 3 m (75 m²);Reach, tangential: 25 x 3 m (75 m²);Transmitter power: < 1 mW;Functions: Group parameterisation, Semi-/fully automatic, Lighting scenario, Manual ON / ON-OFF, Neighbouring group function, Normal / test mode;Twilight setting: 2 – 2000 lx;Time setting: 10 sec – 60 min;Basic light level function: No;Main light adjustable: Yes;Twilight setting TEACH: Yes;Constant-lighting control: No;Interconnection: Yes;Interconnection via: Bluetooth Mesh;Product category: Presence detector

Manufacturer Steinel

Prod. No. 086343

Ordering designation Hallway COM2 - Surface wiring

Deliver, install and set ready for operation