STEINEL Professional Presence detector

Hallway

IP - surface

Dimensions (L x W x H): 62 x 123 x 123 mm;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Bluetooth, Bus;With remote control: No;Version: IP - surface;PU1, EAN: 4007841086404;Type: Presence detector;Application, place: Indoors;Application, room: Indoors, corridor / aisle;Colour: white;Colour, RAL: 9003;Includes corner wall mount: No;Installation site: ceiling;Installation: Surface wiring, Ceiling;IP-rating: IP20;Ambient temperature: from -20 up to 50 °C;Material: Plastic;Mains power supply: 18 – 55 V;Power supply, detail: Passive PoE SELV, standard PoE (IEEE 802.3 af);With bus coupling: Yes;Technology, sensors: High frequency, Mixed light metering, Temperature, Air humidity;Voltage range (min): 18 V;Voltage range (max): 55 V;Mounting height: 2 – 4 m;Mounting height max.: 4,00 m;Optimum mounting height: 2,8 m;HF-system: 5,8 GHz;Detection: also through glass, wood and stud walls;Detection angle: Aisle, 360 °;Angle of aperture: 160 °;Sneak-by guard: Yes;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: 25 x 3 m (75 m²);Reach, tangential: 25 x 3 m (75 m²);Transmitter power: < 1 mW;Twilight setting: 2 – 2000 lx;Basic light level function: No;Main light adjustable: No;Twilight setting TEACH: Yes;Constant-lighting control: No;Interconnection: Yes;Interconnection via: Bluetooth Mesh;Product category: Presence detector

Manufacturer Steinel

Prod. No. 086404

Ordering designation Hallway IP - surface

Deliver, install and set ready for operation