STEINEL Professional Presence detector

Hallway

KNX - concealed black V3.5

Dimensions (L x W x H): 67 x 103 x 103 mm;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Bluetooth Mesh;With remote control: No;Version: KNX - concealed black;PU1, EAN: 4007841086480;Type: Presence detector;Application, place: Indoors;Application, room: Indoors, corridor / aisle;Colour: black;Colour, RAL: 9005;Includes corner wall mount: No;Installation site: ceiling;Installation: Concealed wiring;IP-rating: IP20;Ambient temperature: from -25 up to 50 °C;Material: Plastic;With bus coupling: Yes;Technology, sensors: High frequency;Mounting height: 2 – 4 m;Mounting height max.: 4,00 m;Optimum mounting height: 2,8 m;HF-system: 5,8 GHz;Detection: also through glass, wood and stud walls;Detection angle: 360 °, Aisle;Angle of aperture: 160 °;Sneak-by guard: Yes;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: 25 x 3 m (75 m²);Reach, tangential: 25 x 3 m (75 m²);Transmitter power: < 1 mW;Twilight setting: 2 – 2000 lx;Time setting: 0 sec – 1092 min;Basic light level function: Yes;Main light adjustable: Yes;Twilight setting TEACH: Yes;Interconnection: Yes;Interconnection via: Bluetooth Mesh;Rated current: 30 mA;KNX Medium: TP 256;KNX Secure: No;Product category: Presence detector

Manufacturer Steinel

Prod. No. 086480

Ordering designation Hallway KNX - concealed black

Deliver, install and set ready for operation