STEINEL Professional Presence detector

LuxMaster Hallway

DALI-2 APC - concealed

Dimensions (L x W x H): 67 x 103 x 103 mm;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Bluetooth Mesh;With remote control: No;Version: DALI-2 APC - concealed;PU1, EAN: 4007841086305;Type: Presence detector;Application, place: Indoors;Application, room: Indoors, corridor / aisle;Colour: white;Colour, RAL: 9003;Includes corner wall mount: No;Installation site: ceiling;Installation: Concealed wiring, Ceiling;IP-rating: IP20;Ambient temperature: from -20 up to 50 °C;Material: Plastic;Mains power supply: 220 – 240 V / 50 – 60 Hz;Power supply, detail: max. 64 users;Steuerausgang, Dali: Addressable / broadcast 64 electronic ballasts;Technology, sensors: High frequency, Photo-cell controller;Mounting height: 2 – 4 m;Mounting height max.: 4,00 m;Optimum mounting height: 2,8 m;HF-system: 5,8 GHz;Detection: also through glass, wood and stud walls;Detection angle: Aisle, 360 °;Angle of aperture: 160 °;Sneak-by guard: Yes;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: 25 x 3 m (75 m²);Reach, tangential: 25 x 3 m (75 m²);Transmitter power:

Manufacturer Steinel

Prod. No. 086305

Ordering designation LuxMaster Hallway DALI-2 APC - concealed

Deliver, install and set ready for operation