STEINEL Professional Presence detector

HF 360-2

DALI-2 APC - concealed

Dimensions (L x W x H): 61 x 103 x 103 mm;Manufacturer's Warranty: 5 years;Settings via: Bluetooth Mesh;With remote control: No;Version: DALI-2 APC - concealed;PU1, EAN: 4007841086596;Type: Presence detector;Application, place: Indoors;Application, room: function room / ancillary room, kitchenette, stairwell, WC / washroom, Indoors;Colour: white;Colour, RAL: 9003;Includes corner wall mount: No;Installation site: ceiling;Installation: Concealed wiring, Ceiling;IP-rating: IP20;Ambient temperature: from -25 up to 50 °C;Material: Plastic;Mains power supply: 220 – 240 V / 50 – 60 Hz;Power supply, detail: max. 64 users;Steuerausgang, Dali: Addressable / broadcast 64 electronic ballasts;Technology, sensors: High frequency, Photo-cell controller;Mounting height: 2,5 – 4 m;Mounting height max.: 4,00 m;Optimum mounting height: 2,8 m;HF-system: 5,8 GHz;Detection: also through glass, wood and stud walls;Detection angle: 360 °;Angle of aperture: 160 °;Sneak-by guard: Yes;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: Ø 12 m (113 m²);Reach, tangential: Ø 12 m (113 m²);Transmitter power: < 1 mW;Functions: Semi-/fully automatic, Constant-lighting control, Lighting scenario, Master/slave override, Neighbouring group function, TouchDIM function;Twilight setting: 2 – 2000 lx;Time setting: 30 sec – 60 min;Basic light level function: Yes;Basic light level function time: 10 min, all day, all night;Main light adjustable: Yes;Twilight setting TEACH: Yes;Constant-lighting control: Yes;Interconnection: Yes;Interconnection via: Bluetooth Mesh;Product category: Presence detector

Manufacturer Steinel

Prod. No. 086596

Ordering designation HF 360-2 DALI-2 APC - concealed

Deliver, install and set ready for operation