STEINEL Professional Presence detector

HF 360-2

DALI-2 Input Device - concealed

Dimensions (L x W x H): 61 x 103 x 103 mm;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Bluetooth Mesh;With remote control: No;Version: DALI-2 Input Device - concealed;PU1, EAN: 4007841086619;Type: Presence detector;Application, place: Indoors;Application, room: function room / ancillary room, kitchenette, stairwell, WC / washroom, Indoors;Colour: white;Colour, RAL: 9003;Includes corner wall mount: No;Installation site: ceiling;Package content: 1;Installation: Concealed wiring, Ceiling;IP-rating: IP20;Ambient temperature: from -25 up to 50 °C;Material: Plastic;Mains power supply: 12 – 23 V;Power supply, detail: DALI bus;With bus coupling: Yes;Technology, sensors: High frequency, Light sensor, Temperature, Air humidity;Mounting height: 2,5 – 4 m;Mounting height max.: 4,00 m;Optimum mounting height: 2,8 m;HF-system: 5,8 GHz;Detection: also through glass, wood and stud walls;Detection angle: 360 °;Angle of aperture: 160 °;Sneak-by guard: Yes;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: Ø 12 m (113 m²);Reach, tangential: Ø 12 m (113 m²);Transmitter power: < 1 mW;Twilight setting: 2 – 2000 lx;Basic light level function: No;Main light adjustable: No;Twilight setting TEACH: No;Constant-lighting control: No;Interconnection: Yes;Interconnection via: Bluetooth Mesh;Max. Current consumption according to IEC 62386-101: 84 mA;Typical current consumption at 16V input voltage: 21 mA;Max. Current consumption at 10V input voltage in regular operation: 38 mA;Product category: Presence detector

Manufacturer Steinel

Prod. No. 086619

Ordering designation HF 360-2 DALI-2 Input Device - concealed

Deliver, install and set ready for operation