STEINEL Professional Presence detector

IR Quattro HD-2 24m

KNX - in-ceiling installation black V3.5

Dimensions (L x W x H): 106 x 103 x 103 mm;Manufacturer's Warranty: 5 years;Settings via: Bus, ETS software, App, Potentiometers, Bluetooth Mesh;With remote control: No;Version: KNX - in-ceiling installation black;PU1, EAN: 4007841094553;Type: Presence detector;Application, place: Indoors;Application, room: lecture hall, classroom, open-plan office, high-bay warehouse, production facilities, conference room / meeting room, duty room, recreation room, changing room, reception / lobby, sports hall, multi-storey / underground car park, dining hall / staff restaurant, Indoors;Colour: black;Colour, RAL: 9005;Includes corner wall mount: No;Installation site: ceiling;Installation: Ceiling, In-ceiling installation;IP-rating: IP20;Ambient temperature: from 0 up to 40 °C;Material: Plastic;Mains power supply: 30 V;Power supply, detail: KNX bus;With bus coupling: Yes;Technology, sensors: Mixed light metering, passive infrared, Air humidity, Temperature;Voltage range (min): 30 V;Voltage range (max): 30 V;Mounting height: 2,50 – 10,00 m;Mounting height max.: 10,00 m;Optimum mounting height: 2,8 m;Detection angle: 360 °;Capability of masking out individual segments: Yes;Electronic scalability: Yes;Mechanical scalability: Yes;Reach, radial: 9 x 9 m (81 m²);Reach, tangential: 24 x 24 m (576 m²);Reach, presence: 9 x 9 m (81 m²);Twilight setting: 2 – 2000 lx;Time setting: 10 sec – 1092 min;Basic light level function: Yes;Main light adjustable: Yes;Twilight setting TEACH: Yes;Constant-lighting control: Yes;KNX functions: Comfort, Photo-cell controller, Individual pyro evaluation, Basic light level function, Light level, HVAC output, Constant-lighting control, Light output 4x, Humidity output, Presence output, Day / night function, Dewpoint, Temperature output, Logic gate;Interconnection: Yes;Interconnection via: KNX bus, Bluetooth Mesh;Rated current: 10 mA;KNX Medium: TP 256;KNX Secure: No;Product category: Presence detector

Manufacturer Steinel

Prod. No. 094553

Ordering designation IR Quattro HD-2 24m KNX - in-ceiling installation black

Deliver, install and set ready for operation