STEINEL Professional Sensor-switched LED indoor light

RS PRO 6100 circular SC

with motion detector

Dimensions (L x W x H): 1200 x 90 x 80 mm;With lamp: Yes, STEINEL LED system;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: App, Bluetooth, Bluetooth Mesh;With remote control: No;Version: with motion detector;PU1, EAN: 4007841089832;Application, place: Indoors;Application, room: corridor / aisle, function room / ancillary room, warehouse, multi-storey / underground car park, production facilities, stairwell, changing room, WC / washroom;Colour: grey;Installation site: ceiling, wall;Installation: Surface wiring, Ceiling, Wall;Impact resistance: IK08;IP-rating: IP65;Protection class: I;Ambient temperature: from -20 up to 40 °C;Housing material: Plastic;Cover material: PC structured;Mains power supply: 220 – 240 V / 50 – 60 Hz;Power consumption: 0,3 W;Mounting height max.: 3,50 m;HF-system: 24 GHz;Slave modeselectable: Yes;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: Ø 8 m (50 m²);Reach, tangential: Ø 8 m (50 m²);Photo-cell controller: Yes;Transmitter power: < 1 mW;Colour temperature: 4000 K;Colour variation LED: SDCM3;Lamp: LED can be replaced;Base: without;LED cooling system: Passive Thermo Control;Soft light start: Yes;Continuous light: selectable;Functions: Motions sensor, DIM function, Dimmkurve individuell einstellbar, Adjustable fade time when switching on and off, Semi-/fully automatic, Light sensor, Neighbouring group function, Encrypted communication, Timer;Twilight setting: 2 – 2000 lx;Time setting: 5 sec – 60 min;Basic light level function: Yes;Basic light level function time: 5s-60 min, Brightness-controlled;Main light adjustable: 0 - 100 %;Twilight setting TEACH: Yes;Interconnection: Yes;Type of interconnection: Master/master;Interconnection via: Bluetooth Mesh;Basic light level function in per cent: 10 – 100 %;Output: 28 W;Angle of aperture: 160 °;Detection angle: 360 °;Product category: Sensor-switched LED indoor light

Manufacturer Steinel

Prod. No. 089832

Ordering designation RS PRO 6100 circular SC with motion detector

Deliver, install and set ready for operation