STEINEL Professional Motion detector

IS 345

DALI-2 APC - in-ceiling installation, rd.

Dimensions (Ø x H): 124 x 121 mm;Manufacturer's Warranty: 5 years;Settings via: Remote control, Potentiometers, Smart Remote;With remote control: No;Version: DALI-2 APC - in-ceiling installation, rd.;PU1, EAN: 4007841080655;Type: Motion detectors;Application, place: Indoors;Application, room: corridor / aisle, multi-storey / underground car park, Indoors;Colour: white;Colour, RAL: 9003;Includes corner wall mount: No;Installation site: ceiling;Package content: 1;Installation: In-ceiling installation, Ceiling;IP-rating: IP20;Ambient temperature: from -20 up to 50 °C;Material: Plastic;Mains power supply: 220 – 240 V / 50 – 60 Hz;Steuerausgang, Dali: Broadcast 1x30 electronic ballasts;Technology, sensors: passive infrared, Light sensor;Mounting height: 2,50 – 5,00 m;Mounting height max.: 5,00 m;Optimum mounting height: 2,8 m;Detection angle: Aisle, 360 °;Angle of aperture: 45 °;Sneak-by guard: Yes;Capability of masking out individual segments: Yes;Electronic scalability: No;Mechanical scalability: No;Reach, radial: 12 x 6 m (72 m²);Reach, tangential: 23 x 6 m (138 m²);Switching zones: 280 switching zones;Functions: Normal / test mode, Manual ON / ON-OFF, Constant-lighting control ON-OFF;Twilight setting: 2 – 1000 lx;Time setting: 5 sec – 15 min;Basic light level function: Yes;Basic light level function time: 1-30 min, all night;Main light adjustable: 50 - 100 %;Twilight setting TEACH: Yes;Constant-lighting control: Yes;Interconnection: Yes;Type of interconnection: Master/slave;Interconnection via: Cable;Product category: Motion detector

Manufacturer Steinel

Prod. No. 080655

Ordering designation IS 345 DALI-2 APC - in-ceiling installation, rd.

Deliver, install and set ready for operation