STEINEL DIY Motion detector

IS 130-2

Anthracite

Dimensions (L x W x H): 87 x 78 x 110 mm;Manufacturer's Warranty: 3 years;Settings via: Potentiometers;With remote control: No;Version: Anthracite;PU1, EAN: 4007841035761;Type: Motion detectors;Application, place: Outdoors, Indoors;Application, room: outdoors, front door, all round the building, terrace / balcony, courtyard & driveway;Colour: Anthracite;Colour, RAL: 7016;Includes corner wall mount: No;Installation site: wall;Package content: 1;Installation: Surface wiring, Wall;IP-rating: IP54;Ambient temperature: from -20 up to 50 °C;Material: Plastic;Mains power supply: 230 – 240 V / 50 Hz;Switching output 1, resistive: 600 W;Fluorescent lamps, electronic ballast: 250 W;Fluorescent lamps, uncorrected: 500 VA;Fluorescent lamps, series-corrected: 500 VA;Fluorescent lamps, parallel-corrected: 500 VA;Switching output 1, low-voltage halogen lamps: 500 VA;LED lamps < 2 W: 100 W;LED lamps > 2 W < 8 W: 125 W;LED lamps > 8 W: 250 W;Capacitive load in μF: 88 µF;Technology, sensors: passive infrared, Mixed light metering;Mounting height: 1,80 – 3,00 m;Mounting height max.: 3,00 m;Optimum mounting height: 2 m;Detection angle: 130 °;Angle of aperture: 40 °;Sneak-by guard: Yes;Capability of masking out individual segments: Yes;Electronic scalability: No;Mechanical scalability: No;Reach, radial: r = 3 m (10 m²);Reach, tangential: r = 12 m (163 m²);Switching zones: 260 switching zones;Twilight setting: 2 – 1000 lx;Time setting: 5 sec – 35 min;Basic light level function: No;Main light adjustable: No;Twilight setting TEACH: No;Constant-lighting control: No;Interconnection: Yes;Type of interconnection: Master/master;Interconnection via: Cable;Product category: Motion detector

Manufacturer Steinel

Prod. No. 035761

Ordering designation IS 130-2 Anthracite

Deliver, install and set ready for operation