STEINEL DIY Motion detector

IS 180 digi

Anthracite

Dimensions (L x W x H): 42 x 116 x 116 mm;With motion detector: Yes;Manufacturer's Warranty: 3 years;Settings via: Bluetooth, Smartphone, Tablet;With remote control: No;Version: Anthracite;PU1, EAN: 4007841067274;Type: Motion detectors;Application, place: Outdoors;Application, room: outdoors, garden, front door, courtyard & driveway, all round the building, terrace / balcony, corridor / aisle, Indoors;Colour: Anthracite;Colour, RAL: 7039;Includes corner wall mount: No;Installation site: wall;Package content: 1;Installation: Surface wiring, Wall;IP-rating: IP54;Ambient temperature: from -20 up to 40 °C;Mains power supply: 220 – 240 V / 50 – 60 Hz;Switching output 1, resistive: 1000 W;Fluorescent lamps, electronic ballast: 430 W;Fluorescent lamps, uncorrected: 500 VA;Fluorescent lamps, series-corrected: 400 VA;Fluorescent lamps, parallel-corrected: 400 VA;Switching output 1, low-voltage halogen lamps: 1000 VA;LED lamps < 2 W: 16 W;LED lamps > 2 W < 8 W: 64 W;LED lamps > 8 W: 64 W;Capacitive load in μF: 88 µF;Power consumption: 0,6 W;Technology, sensors: Light sensor, passive infrared;Mounting height: 1,80 – 2,5 m;Mounting height max.: 2,50 m;Optimum mounting height: 2 m;Detection angle: 180 °;Angle of aperture: 90 °;Sneak-by guard: Yes;Capability of masking out individual segments: Yes;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: r = 6 m (57 m²);Reach, tangential: r = 8 m (101 m²);Functions: Motions sensor, Adjustable fade time when switching on and off, Capture zone definition, Feedback indicator, Night economy mode, Normal / test mode, Neighbouring group function, Relay output, Light sensor;Twilight setting: 2 – 1000 lx;Time setting: 5 sec – 60 min;Basic light level function: No;Main light adjustable: No;Twilight setting TEACH: Yes;Constant-lighting control: No;Interconnection via: Bluetooth Mesh;Product category: Motion detector

Manufacturer Steinel

Prod. No. 067274

Ordering designation IS 180 digi Anthracite

Deliver, install and set ready for operation