STEINEL DIY Sensor-switched LED indoor light

RS 20 S

Dimensions (Ø x H): 280 x 110 mm;With lamp: Yes, STEINEL LED system;With motion detector: Yes;Manufacturer's Warranty: 3 years;Settings via: Potentiometers;With remote control: No;PU1, EAN: 4007841079734;Application, place: Indoors;Application, room: corridor / aisle, changing room, function room / ancillary room, kitchenette, stairwell, WC / washroom, outdoors, Indoors;Colour: white;Installation site: wall, ceiling;Installation: Wall, Ceiling, Surface wiring;Impact resistance: IK03;IP-rating: IP44;Protection class: II;Ambient temperature: from 10 up to 40 °C;Housing material: Plastic;Cover material: Plastic, opal;Mains power supply: 220 – 240 V / 50 – 60 Hz;Power consumption: 0,38 W;Mounting height max.: 4,00 m;HF-system: 5,8 GHz;Detection: also through glass, wood and stud walls;Capability of masking out individual segments: Yes;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: Ø 8 m (50 m²);Reach, tangential: Ø 8 m (50 m²);Photo-cell controller: Yes;Transmitter power: < 1 mW;Luminous flux total product: 942 lm;Colour temperature: 3000 K;Colour variation LED: SDCM3;Lamp: LED cannot be replaced;Base: without;LED cooling system: Passive Thermo Control;Soft light start: Yes;Continuous light: selectable, 4h;Twilight setting: 2 – 2000 lx;Time setting: 5 sec – 15 min;Basic light level function: Yes;Basic light level function time: 10/30 min, all night;Twilight setting TEACH: No;Interconnection: Yes;Type of interconnection: Master/master;Interconnection via: Cable;Fuse protection B10: 54;Fuse protection B16: 87;Fuse protection C10: 90;Fuse protection C16: 145;Basic light level function in per cent: 10 %;Output: 9,4 W;Angle of aperture: 160 °;IP-rating, ceiling: IP44;Detection angle: 360 °;Product category: Sensor-switched LED indoor light

Manufacturer Steinel

Prod. No. 079734

Ordering designation RS 20 S

Deliver, install and set ready for operation