STEINEL DIY Sensor-switched LED indoor light

RS 200 SC

with motion detector & Bluetooth

Dimensions (Ø x H): 263 x 53 mm;With lamp: Yes, STEINEL LED system;With motion detector: Yes;Manufacturer's Warranty: 3 years;Settings via: Smartphone, Tablet, App, Bluetooth Mesh;With remote control: No;Version: with motion detector & Bluetooth;PU1, EAN: 4007841078782;Application, place: Indoors;Application, room: stairwell, Indoors, function room / ancillary room, corridor / aisle, recreation room;Colour: Anthracite;Package content: 1;Installation site: wall, ceiling;Installation: Ceiling, Wall;Impact resistance: IK10;IP-rating: IP54;Protection class: I;Ambient temperature: from -20 up to 40 °C;Housing material: Aluminium;Cover material: Plastic, opal;Mains power supply: 220 – 240 V / 50 – 60 Hz;Mounting height max.: 4,00 m;HF-system: 5,8 GHz;Slave modeselectable: Yes;Detection: also through glass, wood and stud walls;Electronic scalability: Yes;Reach, radial: Ø 8 m (50 m²);Reach, tangential: Ø 8 m (50 m²);Photo-cell controller: Yes;Transmitter power: < 1 mW;Luminous flux total product: 1165 lm;Total product efficiency: 68 lm/W;Colour temperature: 3000 K;Colour variation LED: SDCM3;Lamp: LED cannot be replaced;LED cooling system: Passive Thermo Control;Soft light start: Yes;Continuous light: selectable, 4h;Functions: Motions sensor, DIM function, Light sensor, Orientation light;Twilight setting: 2 – 2000 lx;Time setting: 5 sec – 30 min;Basic light level function: Yes;Basic light level function, detail: LED effect light;Basic light level function time: 1-60 min;Main light adjustable: 50 - 100 %;Interconnection: Yes;Type of interconnection: Master/master;Interconnection via: Bluetooth Mesh;Lifetime LED according to IEC-62717 (L70): 54000;LED service life according to IES TM-21 (L70): 54000;Fuse protection B10: 134;Fuse protection B16: 215;Fuse protection C10: 134;Fuse protection C16: 215;Basic light level function in per cent: 10 – 100 %;Output: 17,1 W;Colour Rendering Index CRI: = 80;Photobiological safety in accordance with EN 62471: RG1;Angle of aperture: 160 °;Detection angle: 360 °;Product category: Sensor-switched LED indoor light

Manufacturer Steinel

Prod. No. 078782

Ordering designation RS 200 SC with motion detector & Bluetooth

Deliver, install and set ready for operation