STEINEL DIY Sensor-switched LED outdoor light

L 240 S

Anthracite

Dimensions (L x W x H): 81 x 165 x 305 mm;With lamp: Yes, STEINEL LED system;Manufacturer's Warranty: 3 years;Settings via: Potentiometers;With remote control: No;Version: Anthracite;PU1, EAN: 4007841056506;Application, place: Outdoors;Application, room: outdoors, front door, all round the building, terrace / balcony, courtyard & driveway;Colour: Anthracite;Package content: 1;Installation site: wall;Impact resistance: IK07;IP-rating: IP44;Protection class: II;Ambient temperature: from -20 up to 50 °C;Housing material: Stainless steel;Cover material: Plastic, opal;Mains power supply: 220 – 240 V / 50 – 60 Hz;Output: 9,3 W;Average rated life expectancy of power supply unit at 25°C: > 60000 h;Mounting height max.: 3,00 m;Slave modeselectable: No;Sneak-by guard: Yes;Capability of masking out individual segments: Yes;Electronic scalability: No;Mechanical scalability: No;Reach, radial: r = 3 m (14 m²);Reach, tangential: r = 10 m (157 m²);Photo-cell controller: Yes;Cover material: sticker;Luminous flux total product: 946 lm;Measured luminos flux (360°): 946 lm;Total product efficiency: 101 lm/W;Colour temperature: 3000 K;Colour variation LED: SDCM3;Lamp: LED cannot be replaced;LED life expectancy (max. °C): 60000 h;Service life LED L70B50 (25°): > 60000 h;Base: without;LED cooling system: Passive Thermo Control;Soft light start: No;Twilight setting: 2 – 2000 lx;Time setting: 8 sec – 35 min;Basic light level function: No;Main light adjustable: No;Interconnection: No;Colour Rendering Index CRI: = 82;Optimum mounting height: 2 m;Detection angle: 180 °;Product category: Sensor-switched LED outdoor light

Manufacturer Steinel

Prod. No. 056506

Ordering designation L 240 S Anthracite

Deliver, install and set ready for operation