STEINEL DIY Sensor-switched LED outdoor light

L 271 digi SC

with motion detector & Bluetooth

Dimensions (L x W x H): 65 x 116 x 235 mm;With lamp: Yes, STEINEL LED system;Manufacturer's Warranty: 3 years;Settings via: App, Smartphone, Tablet, Bluetooth Mesh;With remote control: No;Version: with motion detector & Bluetooth;PU1, EAN: 4007841067212;Application, place: Outdoors;Application, room: garden, front door, all round the building, terrace / balcony;Colour: Anthracite;includes sheet of self-adhesive numbers: No;Package content: 1;Installation site: wall;Impact resistance: IK03;IP-rating: IP44;Protection class: II;Ambient temperature: from -10 up to 40 °C;Housing material: Aluminium;Cover material: Plastic, opal;Mains power supply: 220 – 240 V / 50 – 60 Hz;Output: 6,6 W;Average rated life expectancy of power supply unit at 25°C: > 60000 h;Mounting height max.: 2,50 m;Slave modeselectable: Yes;Sneak-by guard: Yes;Capability of masking out individual segments: Yes;Electronic scalability: Yes;Mechanical scalability: No;Reach, detail: 3 detection zones can be (de)activated individually, range can be set separately;Reach, radial: r = 6 m (57 m²);Reach, tangential: r = 8 m (101 m²);Luminous flux total product: 566 lm;Measured luminos flux (360°): 566 lm;Total product efficiency: 85 lm/W;Colour temperature: 3000 K;Lamp: LED cannot be replaced;Service life LED L70B50 (25°): > 60000 h;Soft light start: Yes;Continuous light: selectable;Twilight setting: 2 – 2000 lx;Time setting: 5 sec – 60 min;Basic light level function: Yes;Basic light level function, detail: LED effect light;Basic light level function time: 10s-60 min, OFF, Brightness-controlled;Main light adjustable: 5 - 100 %;Twilight setting TEACH: Yes;Interconnection: Yes;Basic light level function in per cent: 5 – 100 %;Basic light level function percentage, from: 5 %;Basic light level function percentage, up to: 100 %;Interconnection via: Bluetooth Mesh;Colour Rendering Index CRI: = 82;Optimum mounting height: 2 m;Detection angle: 180 °;Product category: Sensor-switched LED outdoor light

Manufacturer Steinel

Prod. No. 067212

Ordering designation L 271 digi SC with motion detector & Bluetooth

Deliver, install and set ready for operation