STEINEL DIY Sensor-switched LED outdoor light

L 605 S

with motion detector

Dimensions (L x W x H): 131 x 78 x 260 mm;With lamp: Yes, STEINEL LED system;Manufacturer's Warranty: 3 years;Settings via: Potentiometers;With remote control: No;Version: with motion detector;PU1, EAN: 4007841065287;Application, place: Outdoors;Application, room: outdoors, front door, all round the building, terrace / balcony;Colour: Anthracite;includes sheet of self-adhesive numbers: No;Package content: 1;Installation site: wall;Impact resistance: IK03;IP-rating: IP44;Protection class: I;Ambient temperature: from -10 up to 40 °C;Housing material: Aluminium;Cover material: Glass opal;Mains power supply: 220 – 240 V / 50 – 60 Hz;Output: 11,3 W;Life expectancy of power supply unit (max. °C): 36000 h;Average rated life expectancy of power supply unit at 25°C: > 36000 h;Mounting height max.: 2,00 m;Slave modeselectable: No;Sneak-by guard: No;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: r = 2 m (6 m²);Reach, tangential: r = 10 m (157 m²);Photo-cell controller: Yes;Cover material: sticker;Luminous flux total product: 729 lm;Measured luminos flux (360°): 729 lm;Total product efficiency: 64,5 lm/W;Colour temperature: 3000 K;Lamp: LED cannot be replaced;LED life expectancy (max. °C): 36000 h;Service life LED L70B50 (25°): > 36000 h;Base: without;LED cooling system: Passive Thermo Control;Soft light start: Yes;Continuous light: selectable;Twilight setting: 10 – 2000 lx;Time setting: 5 sec – 15 min;Basic light level function: Yes;Main light adjustable: No;Twilight setting TEACH: No;Interconnection: No;Basic light level function in per cent: 10 %;Basic light level function percentage, from: 10 %;Basic light level function percentage, up to: 10 %;Colour Rendering Index CRI: = 82;Optimum mounting height: 2 m;Detection angle: 180 °;Product category: Sensor-switched LED outdoor light

Manufacturer Steinel

Prod. No. 065287

Ordering designation L 605 S with motion detector

Deliver, install and set ready for operation