STEINEL DIY Sensor-switched LED outdoor light

L 835 SC

Anthracite

Dimensions (L x W x H): 132 x 110 x 110 mm;With lamp: Yes, STEINEL LED system;Manufacturer's Warranty: 3 years;Settings via: Bluetooth Mesh, App, Smartphone, Tablet;With remote control: No;Version: Anthracite;PU1, EAN: 4007841055516;Application, place: Outdoors;Application, room: outdoors, garden, front door, courtyard & driveway, all round the building, terrace / balcony;Colour: Anthracite;includes sheet of self-adhesive numbers: No;Package content: 1;Installation site: wall;Impact resistance: IK03;IP-rating: IP44;Protection class: II;Ambient temperature: from -20 up to 40 °C;Housing material: Plastic;Cover material: Plastic, opal;Mains power supply: 220 – 240 V / 50 – 60 Hz;Output: 9,1 W;Mounting height max.: 2,50 m;Slave modeselectable: Yes;Capability of masking out individual segments: Yes;Electronic scalability: Yes;Mechanical scalability: Yes;Reach, detail: Detection zone adjustable on 2 axes;Reach, radial: r = 5 m (35 m²);Reach, tangential: r = 5 m (35 m²);Photo-cell controller: Yes;Luminous flux total product: 616 lm;Measured luminos flux (360°): 616 lm;Total product efficiency: 68 lm/W;Colour temperature: 3000 K;Lamp: LED cannot be replaced;LED life expectancy (max. °C): 50000 h;Service life LED L70B50 (25°): > 60000 h;Base: without;LED cooling system: Passive Thermo Control;Soft light start: Yes;Continuous light: selectable;Functions: Motions sensor, Selecting programmed lighting scenarios, Night economy mode, Lighting scenario;Twilight setting: 2 – 2000 lx;Time setting: 5 sec – 60 min;Basic light level function: Yes;Basic light level function time: 10 sec - 60 min;Main light adjustable: 0 - 100 %;Twilight setting TEACH: Yes;Interconnection: Yes;Basic light level function in per cent: 0 – 100 %;Basic light level function percentage, from: 0 %;Basic light level function percentage, up to: 100 %;Interconnection via: Bluetooth Mesh;Colour Rendering Index CRI: = 82;Optimum mounting height: 2 m;Detection angle: 160 °;Product category: Sensor-switched LED outdoor light

Manufacturer Steinel

Prod. No. 055516

Ordering designation L 835 SC Anthracite

Deliver, install and set ready for operation