STEINEL DIY Sensor-switched LED floodlight

XLED CAM2 SC

Anthracite

Dimensions (L x W x H): 167 x 210 x 275 mm;With lamp: Yes, LED lamp;With motion detector: Yes;Manufacturer's Warranty: 3 years;Settings via: Wi-Fi;Version: Anthracite;PU1, EAN: 4007841092603;Application, place: Outdoors;Application, room: outdoors, all round the building, terrace / balcony, courtyard & driveway;Colour: Anthracite;Includes corner wall mount: No;Installation site: wall;Installation: Wall, Surface wiring;Impact resistance: IK03;IP-rating: IP44;Protection class: I;Ambient temperature: from -10 up to 40 °C;Housing material: Aluminium;Cover material: Glass transparent;Mains power supply: 220 – 240 V / 50 – 60 Hz;Mounting height max.: 2,00 m;Sneak-by guard: No;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: r = 2 m (6 m²);Reach, tangential: r = 10 m (157 m²);Photo-cell controller: Yes;Luminous flux total product: 2310 lm;Measured luminos flux (360°): 2310 lm;Total product efficiency: 110 lm/W;Colour temperature: 3000 K;Colour variation LED: SDCM3;Lamp: LED cannot be replaced;LED life expectancy (max. °C): 50000 h;Service life LED L70B50 (25°): > 36000 h;LED cooling system: Passive Thermo Control;Soft light start: No;Continuous light: selectable;Functions: Iintercom system, Camera;Twilight setting: 10 – 10000 lx;Time setting: 60 sec – 15 min;Basic light level function: Yes;Basic light level function time: all night;Main light adjustable: 30 - 100 %;Twilight setting TEACH: No;Interconnection via: Wi-Fi;Basic light level function in per cent: 0 – 40 %;Basic light level function percentage, from: 0 %;Basic light level function percentage, up to: 40 %;Output: 21 W;Colour Rendering Index CRI: = 80;Optimum mounting height: 2 m;Detection angle: 180 °;App: STEINEL CAM App 2;Voraussetzung für Inbetriebnahme: 2,4 GHz, 5 Ghz, Wi-Fi 4, Wi-Fi 5, Wi-Fi 6, WPA2/ WPA3, > 2 Mbit/s;Product category: Sensor-switched LED floodlight;Energy efficiency class of light source: E

Manufacturer Steinel

Prod. No. 092603

Ordering designation XLED CAM2 SC Anthracite

Deliver, install and set ready for operation