STEINEL DIY LED floodlight without sensor

XLED ONE XL

Anthracite

Dimensions (L x W x H): 193 x 259 x 215 mm;With lamp: Yes, STEINEL LED system;With motion detector: No;Manufacturer's Warranty: 3 years;Version: Anthracite;PU1, EAN: 4007841065225;Application, place: Outdoors;Application, room: outdoors, courtyard & driveway;Colour: Anthracite;Installation site: wall, ceiling;Installation: Wall, Ceiling, Surface wiring;Impact resistance: IK03;IP-rating: IP44;Protection class: I;Ambient temperature: from -20 up to 40 °C;Housing material: Aluminium;Cover material: Glass transparent;Mains power supply: 220 – 240 V / 50 – 60 Hz;Photo-cell controller: No;Luminous flux total product: 4200 lm;Measured luminos flux (360°): 4200 lm;Total product efficiency: 99 lm/W;Colour temperature: 3000 K;Colour variation LED: SDCM3;Lamp: LED cannot be replaced;LED life expectancy (max. °C): 50000 h;Service life LED L70B50 (25°): > 36000 h;Drop in luminous flux in accordance with LM80: L70B10;LED cooling system: Passive Thermo Control;Basic light level function: No;Interconnection: No;Output: 42,6 W;Colour Rendering Index CRI: = 80;Product category: LED floodlight without sensor

Manufacturer Steinel

Prod. No. 065225

Ordering designation XLED ONE XL Anthracite

Deliver, install and set ready for operation