STEINEL DIY Sensor-switched LED floodlight

XLED ONE XL S

Anthracite

Dimensions (L x W x H): 222 x 259 x 215 mm;With lamp: Yes, STEINEL LED system;With motion detector: Yes;Manufacturer's Warranty: 3 years;Settings via: Potentiometers;Version: Anthracite;PU1, EAN: 4007841065263;Application, place: Outdoors;Application, room: outdoors, courtyard & driveway;Colour: Anthracite;Installation site: wall, ceiling;Installation: Wall, Ceiling, Surface wiring;Impact resistance: IK03;IP-rating: IP44;Protection class: I;Ambient temperature: from -20 up to 40 °C;Housing material: Aluminium;Cover material: Glass transparent;Mains power supply: 220 – 240 V / 50 – 60 Hz;Mounting height max.: 3,00 m;Sneak-by guard: No;Capability of masking out individual segments: Yes;Electronic scalability: No;Mechanical scalability: No;Reach, radial: r = 3 m (14 m²);Reach, tangential: r = 12 m (226 m²);Photo-cell controller: Yes;Luminous flux total product: 4200 lm;Measured luminos flux (360°): 4200 lm;Total product efficiency: 99 lm/W;Colour temperature: 3000 K;Colour variation LED: SDCM3;Lamp: LED cannot be replaced;LED life expectancy (max. °C): 50000 h;Service life LED L70B50 (25°): > 36000 h;Drop in luminous flux in accordance with LM80: L70B10;LED cooling system: Passive Thermo Control;Twilight setting: 2 – 2000 lx;Time setting: 8 sec – 35 min;Basic light level function: No;Main light adjustable: No;Interconnection: No;Output: 42,6 W;Colour Rendering Index CRI: = 80;Optimum mounting height: 2 m;Detection angle: 180 °;Product category: Sensor-switched LED floodlight

Manufacturer Steinel

Prod. No. 065263

Ordering designation XLED ONE XL S Anthracite

Deliver, install and set ready for operation