STEINEL DIY Sensor-switched LED floodlight

XLED Protect S

with motion detector - anthracite

Dimensions (L x W x H): 195 x 195 x 187 mm;With lamp: Yes, LED lamp;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Potentiometers;Version: with motion detector - anthracite;PU1, EAN: 4007841090012;Application, place: Outdoors;Application, room: garden, front door, courtyard & driveway, all round the building, terrace / balcony;Colour: Anthracite;Includes corner wall mount: No;Installation site: wall, corner;Installation: Wall, Surface wiring;Impact resistance: IK03;IP-rating: IP54;Protection class: II;Ambient temperature: from -20 up to 40 °C;Cover material: Plastic, opal;Mains power supply: 220 – 240 V / 50 – 60 Hz;Mounting height max.: 4,00 m;Slave modeselectable: No;Sneak-by guard: No;Capability of masking out individual segments: No;Electronic scalability: No;Reach, radial: r = 3 m (14 m²);Reach, tangential: r = 10 m (157 m²);Photo-cell controller: No;Cover material: sticker;Luminous flux total product: 1642 lm;Measured luminos flux (360°): 1642 lm;Total product efficiency: 115 lm/W;Colour temperature: 3000 K;Lamp: LED cannot be replaced;Service life LED L70B50 (25°): > 60000 h;Soft light start: No;Functions: Motions sensor, Light sensor, Potentiometer;Twilight setting: 2 – 1000 lx;Time setting: 5 sec – 35 min;Basic light level function: No;Main light adjustable: No;Twilight setting TEACH: No;Interconnection: No;Output: 14,3 W;Colour Rendering Index CRI: = 82;R9-Farbwiedergabeindex: 18;Optimum mounting height: 2 m;Detection angle: 180 °;Product category: Sensor-switched LED floodlight;Energy efficiency class of light source: E

Manufacturer Steinel

Prod. No. 090012

Ordering designation XLED Protect S with motion detector - anthracite

Deliver, install and set ready for operation