STEINEL DIY Motion detector

IR 180 AP easy

Surface wiring

Dimensions (L x W x H): 82 x 72 x 67 mm;With motion detector: Yes;Manufacturer's Warranty: 3 years;Settings via: Potentiometers;With remote control: No;Version: Surface wiring;PU1, EAN: 4007841066048;Type: Motion detectors;Application, place: Indoors, Outdoors;Application, room: corridor / aisle, function room / ancillary room, WC / washroom, outdoors, Indoors;Colour: grey;Colour, RAL: 7035;Includes corner wall mount: No;Installation site: wall;Package content: 1;Installation: Surface wiring, Wall;IP-rating: IP54;Protection class: II;Ambient temperature: from 0 up to 40 °C;Material: Plastic;Mains power supply: 220 – 240 V / 50 – 60 Hz;Switching output 1, resistive: 200 W;Switching output 1, number of LEDs / fluorescent lamps: 4 pcs.;Fluorescent lamps, electronic ballast: 150 W;Fluorescent lamps, uncorrected: 150 VA;Fluorescent lamps, series-corrected: 150 VA;Fluorescent lamps, parallel-corrected: 150 VA;Switching output 1, low-voltage halogen lamps: 200 VA;LED lamps < 2 W: 40 W;LED lamps > 2 W < 8 W: 100 W;LED lamps > 8 W: 160 W;Capacitive load in μF: 132 µF;Technology, sensors: passive infrared, Light sensor;Mounting height: 0,90 – 2,20 m;Mounting height max.: 2,20 m;Optimum mounting height: 1,1 m;Detection angle: 180 °;Angle of aperture: 90 °;Sneak-by guard: Yes;Electronic scalability: No;Mechanical scalability: No;Reach, radial: r = 3 m (14 m²);Reach, tangential: r = 8 m (101 m²);Switching zones: 96 switching zones;Twilight setting: 2 – 1000 lx;Time setting: 5 sec – 30 min;Basic light level function: No;Main light adjustable: No;Twilight setting TEACH: Yes;Constant-lighting control: No;Interconnection: Yes;Product category: Motion detector

Manufacturer Steinel

Prod. No. 066048

Ordering designation IR 180 AP easy Surface wiring

Deliver, install and set ready for operation