STEINEL Professional Presence detector

HF 360-2

COM1 - surface white

Dimensions (L x W x H): 57 x 123 x 123 mm;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Bluetooth Mesh;With remote control: No;Version: COM1 - surface white;PU1, EAN: 4007841086763;Type: Presence detector;Application, place: Indoors;Application, room: function room / ancillary room, kitchenette, stairwell, WC / washroom, Indoors;Colour: white;Colour, RAL: 9003;Includes corner wall mount: No;Installation site: ceiling;Installation: Surface wiring, Ceiling;IP-rating: IP54;Ambient temperature: from -25 up to 50 °C;Material: Plastic;Mains power supply: 220 – 240 V / 50 – 60 Hz;Switching output 1, resistive: 2000 W;Switching output 1, floating: No;Switching output 1, low-voltage halogen lamps: 1000 VA;Power consumption: 0,5 W;Technology, sensors: High frequency, Light sensor;Mounting height: 2,5 – 4 m;Mounting height max.: 4,00 m;Optimum mounting height: 2,8 m;HF-system: 5,8 GHz;Detection: also through glass, wood and stud walls;Detection angle: 360 °;Angle of aperture: 160 °;Sneak-by guard: Yes;Capability of masking out individual segments: No;Electronic scalability: Yes;Mechanical scalability: No;Reach, radial: Ø 12 m (113 m²);Reach, tangential: Ø 12 m (113 m²);Transmitter power: < 1 mW;Functions: Semi-/fully automatic, Master/slave override, Neighbouring group function, Normal / test mode, Presence function, Group parameterisation, Lighting scenario, Manual ON / ON-OFF;Twilight setting: 2 – 2000 lx;Time setting: 10 sec – 60 min;Basic light level function: No;Main light adjustable: No;Twilight setting TEACH: Yes;Constant-lighting control: No;Interconnection: Yes;Interconnection via: Bluetooth Mesh;Product category: Presence detector

Manufacturer Steinel

Prod. No. 086763

Ordering designation HF 360-2 COM1 - surface white

Deliver, install and set ready for operation