STEINEL Professional Motion detector

sensIQ

Anthracite EVO

Dimensions (L x W x H): 172 x 113 x 144 mm;With motion detector: Yes;Manufacturer's Warranty: 5 years;Settings via: Remote control, Potentiometers, Smart Remote;With remote control: Yes;Version: Anthracite;PU1, EAN: 4007841056636;Type: Motion detectors;Application, place: Indoors, Outdoors;Application, room: multi-storey / underground car park, outdoors, warehouse, front door, all round the building, courtyard & driveway, Indoors;Colour: Anthracite;Colour, RAL: 7016;Includes corner wall mount: Yes;Installation site: wall, ceiling, corner;Package content: 1;Installation: Surface wiring, Wall, Ceiling, corner;IP-rating: IP54;Ambient temperature: from -20 up to 50 °C;Material: Plastic;Mains power supply: 230 V / 50 – 60 Hz;Switching output 1, number of LEDs / fluorescent lamps: 50 pcs.;Switching output 1, floating: No;Fluorescent lamps, electronic ballast: 500 W;Fluorescent lamps, series-corrected: 1800 VA;Fluorescent lamps, parallel-corrected: 500 VA;LED lamps < 2 W: 100 W;LED lamps > 2 W < 8 W: 250 W;LED lamps > 8 W: 500 W;Capacitive load in μF: 176 µF;Technology, sensors: passive infrared, Mixed light metering;Voltage range (min): 230 V;Voltage range (max): 230 V;Mounting height: 2,00 – 5,00 m;Mounting height max.: 5,00 m;Optimum mounting height: 2 m;Detection angle: 300 °;Angle of aperture: 180 °;Sneak-by guard: Yes;Capability of masking out individual segments: Yes;Electronic scalability: No;Mechanical scalability: Yes;Reach, radial: r = 4 m (42 m²);Reach, tangential: r = 20 m (1047 m²);Switching zones: 1360 switching zones;Functions: Normal / test mode, Manual ON / ON-OFF;Twilight setting: 2 – 2000 lx;Time setting: 5 sec – 15 min;Basic light level function: No;Main light adjustable: No;Twilight setting TEACH: Yes;Constant-lighting control: No;Interconnection: Yes;Type of interconnection: Master/master;Interconnection via: Cable;Product category: Motion detector

Manufacturer Steinel

Prod. No. 056636

Ordering designation sensIQ Anthracite

Deliver, install and set ready for operation